Comprehensive KPI Threshold Reference Guide - Complete Edition

Based on your request, here is the complete, production-ready KPI threshold reference guide for 2G, 3G, and 4G technologies.

1. KPI Dimensional Framework

Before diving into the KPIs, here's the framework that organizes all metrics into four performance dimensions:

| Dimension | Symbol | Definition | User Impact | Priority |
| --- | --- | --- | --- | --- |
| Accessibility | 🚪 | Ability to successfully access the network and establish services | First impression — if they can't connect, network is perceived as "down" | Highest |
| Retainability | 🔗 | Ability to maintain established connections without dropping | Dropped calls/sessions are the #1 complaint | Highest |
| Mobility | 🚶 | Ability to seamlessly move across cells/frequencies/technologies | Ensures uninterrupted service for moving subscribers | High |
| Resource Utilization | 📊 | Efficiency and availability of network resources | Impacts capacity, congestion, and user experience during peak hours | Medium |

2. GSM (2G) Complete Threshold Table

2.1 Accessibility KPIs (🚪)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TBF Establishment Success Rate (DL) | Downlink TBF Establishment Success Rate(%) | ≥ 95% | >= | 1 | ≥ 95% | ≥ 93% | Success rate of establishing a Downlink Temporary Block Flow for packet data transfer | Failed data sessions, slow internet, poor GPRS/EDGE performance | 3GPP TS 45.008 |
| 2 | SDCCH Congestion Rate | SDCCH Congestion Rate(%) | ≤ 0.5% | <= | 1 | ≤ 0.5% | ≤ 1% | Probability of failure to access a Standalone Dedicated Control Channel during call setup | Cannot make calls, send SMS, or perform location updates | 3GPP TS 45.008 |
| 3 | TCH Congestion Rate | TCH Congestion Rate(%) | ≤ 1.5% | <= | 1 | ≤ 1.5% | ≤ 2% | Probability of failure to access a Traffic Channel during call setup | Users hear busy tones; call setup fails during peak hours | 3GPP TS 45.008 |
| 4 | Call Setup Success Rate - CS | Call Setup Success Rate(%) | ≥ 98% | >= | 2 | ≥ 98% | ≥ 96% | Ratio of successful call establishments to total call attempts | High call setup failures; users cannot connect calls; churn risk | Huawei GSM BSC6000 |
| 5 | Immediate Assignment Success Rate | Immediate Assignment Success Rate(%) | ≥ 97% | >= | 1 | ≥ 97% | ≥ 95% | Success rate of immediate assignment on SDCCH after channel request | Signaling failures; users face delays or failures in call initiation | Huawei GSM BSC6000 |
| 6 | Assignment Success Rate | RCA313:Assignment Success Rate(%) | ≥ 98% | >= | 1 | ≥ 98% | ≥ 96% | Success rate of TCH assignment after SDCCH seizure | Resource allocation failures; impacts all voice services | Huawei GSM BSC6000 |

2.2 Retainability KPIs (🔗)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | TCH Call Drop Rate | TCH Drop Rate(%) | ≤ 1% | <= | 2 | ≤ 1% | ≤ 1.5% | Percentage of calls prematurely terminated before normal release on Traffic Channels | Frequent call interruptions; dropped calls during conversation | 3GPP TS 45.008 |
| 8 | SDCCH Call Drop Rate | SDCCH Drop Rate(%) | ≤ 1% | <= | 1 | ≤ 1% | ≤ 1.5% | Percentage of signaling connections dropped on SDCCH before call setup completes | Call setup failures; dropped connections during call establishment | 3GPP TS 45.008 |

2.3 Mobility KPIs (🚶)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | Handover Success Rate | Handover Success Rate(%) | ≥ 97% | >= | 1 | ≥ 97% | ≥ 95% | Ability to transfer an ongoing call from one cell to another without dropping | Dropped calls during mobility; poor experience for moving subscribers | 3GPP TS 45.008 |
| 10 | Internal Handover Success Rate | CH313/CH310 | ≥ 98% | >= | 1 | ≥ 98% | ≥ 96% | Success rate of internal inter-cell handovers | Poor intra-BSC mobility; affects users moving within same BSC area | Huawei GSM BSC6000 |
| 11 | External Handover Success Rate | CH333/CH330 | ≥ 96% | >= | 1 | ≥ 96% | ≥ 94% | Success rate of external inter-cell handovers | Poor inter-BSC mobility; affects users moving between BSC areas | Huawei GSM BSC6000 |

2.4 Resource Utilization KPIs (📊)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | TCH Availability | RR307:TCH Availability(%) | ≥ 98% | >= | 1 | ≥ 98% | ≥ 95% | Percentage of configured TCHs available for service | Fewer channels for users; impacts capacity and accessibility | Huawei GSM BSC6000 |
| 13 | TRX Availability Rate | TRX Availability Rate(%) | ≥ 98% | >= | 1 | ≥ 98% | ≥ 95% | Percentage of transceivers (TRXs) available for service | Hardware failures cause service degradation; impacts coverage and capacity | Huawei GSM BSC6000 |
| 14 | SDCCH Availability | K3005/K3006 | ≥ 95% | >= | 1 | ≥ 95% | ≥ 93% | Availability of SDCCH channels for signaling | Insufficient signaling channels; impacts SMS and call setup | Huawei GSM BSC6000 |
| 15 | Interference Band Proportion (4~5) | Interference Band Proportion (4~5)(%) | ≤ 5% | <= | 1 | ≤ 5% | ≤ 8% | Proportion of interference measured in bands 4-5 (higher interference) | Degraded voice quality; dropped calls and poor MOS | Huawei GSM BSC6000 |
| 16 | Downlink HQI Proportion (0~5) | Downlink HQI Proportion (0~5)(%) | ≥ 98% | >= | 1 | ≥ 98% | ≥ 95% | Percentage of good quality downlink frames | Voice distortion; impacts user experience | Huawei GSM BSC6000 |
| 17 | Uplink HQI Proportion (0~5) | Uplink HQI Proportion (0~5)(%) | ≥ 98% | >= | 1 | ≥ 98% | ≥ 95% | Percentage of good quality uplink frames | Voice distortion; impacts user experience | Huawei GSM BSC6000 |
| 18 | RF Resource Utilization Rate | RF Resource Utilization Rate(%) | ≤ 70% | <= | 1 | ≤ 70% | ≤ 80% | Utilization of RF resources (TCH + SDCCH) | Congestion; impacts accessibility and retainability | Huawei GSM BSC6000 |
| 19 | Rate of Transmitted Error Frames | RL9A08:Rate of Transmitted Error Frames(%) | ≤ 5% | <= | 1 | ≤ 5% | ≤ 8% | Percentage of error frames transmitted on Abis interface | Voice quality issues; impacts call retainability | Huawei GSM BSC6000 |

3. UMTS (3G) Complete Threshold Table

3.1 Accessibility KPIs (🚪)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | RRC Setup Success Rate | RRC Setup Success Ratio_EFD | ≥ 98.7% | >= | 2 | ≥ 98.7% | ≥ 98% | Ratio of successful Radio Resource Control connection establishments to attempts | UEs cannot establish radio connection; users cannot access any network service | 3GPP TS 25.331 |
| 2 | RAB Setup Success Rate - AMR | AMR RAB Setup Success Rate(%) | ≥ 98.5% | >= | 1 | ≥ 98.5% | ≥ 97.5% | Ratio of successful AMR (voice) Radio Access Bearer establishments | Voice service accessibility degraded; users cannot make voice calls | 3GPP TS 25.413 |
| 3 | RAB Setup Success Rate - PS | PS RAB Setup Success Ratio_EFD | ≥ 97.5% | >= | 1 | ≥ 97.5% | ≥ 96.5% | Ratio of successful PS (data) Radio Access Bearer establishments | Data service accessibility degraded; users cannot browse or use data apps | 3GPP TS 25.413 |
| 4 | Call Setup Success Rate | Call Setup Success Rate(%)_EFD | ≥ 98% | >= | 2 | ≥ 98% | ≥ 97% | Combines RRC and RAB success to measure overall call establishment performance | Widespread call failures; poor network accessibility | Huawei RNC V600R008 |

3.2 Retainability KPIs (🔗)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | RAB Drop Rate - AMR (CS) | CS Call Drop Rate(%) | ≤ 1% | <= | 2 | ≤ 1% | ≤ 1.5% | Ratio of abnormal AMR RAB releases to successful RAB establishments | Voice calls frequently dropped; poor retainability and user dissatisfaction | 3GPP TS 25.413 |
| 6 | RAB Drop Rate - PS | PS Drop Ratio(EFD) | ≤ 3% | <= | 1 | ≤ 3% | ≤ 4% | Ratio of abnormal PS RAB releases to successful PS RAB establishments | Data sessions frequently interrupted; poor data retainability | 3GPP TS 25.413 |

3.3 Mobility KPIs (🚶)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Soft Handover Success Rate | Soft Handover Success Rate(%) | ≥ 99% | >= | 1 | ≥ 99% | ≥ 98.5% | Success rate of "make-before-break" handover (UE maintains multiple simultaneous links) | Increased call drop risk during mobility; essential for UMTS voice quality | 3GPP TS 25.331 |
| 8 | Inter-RAT Handover Success Rate - CS | Inter-RAT Handover Success Rate CS(%) | ≥ 96% | >= | 1 | ≥ 96% | ≥ 94% | Success rate of handover from UMTS to GSM for voice calls | Voice calls drop when moving out of UMTS coverage; poor mobility experience | 3GPP TS 25.331 |
| 9 | Inter-RAT Handover Success Rate - PS | Inter-RAT Handover Success Rate PS(%) | ≥ 80% | >= | 1 | ≥ 80% | ≥ 78% | Success rate of handover from UMTS to GSM for data sessions | Data sessions drop during inter-RAT mobility; users lose connectivity | 3GPP TS 25.331 |
| 10 | Inter-frequency Handover Success Rate | VS.HHO.InterFreqOut.Success Ratio | ≥ 92% | >= | 1 | ≥ 92% | ≥ 90% | Success rate of handover between different UMTS frequencies | Poor mobility between frequency layers; users may drop when moving between layers | Huawei RNC V600R008 |
| 11 | Intra-frequency Handover Success Rate | IntraFreq HHO SR | ≥ 98% | >= | 1 | ≥ 98% | ≥ 96% | Success rate of handover between cells on the same UMTS frequency | Poor mobility within same frequency layer; essential for continuous coverage | Huawei RNC V600R008 |

3.4 Resource Utilization KPIs (📊)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | RNC Availability | Availability | ≥ 99.5% | >= | 1 | ≥ 99.5% | ≥ 99% | Percentage of time RNC is available to manage cells | RNC unavailability affects all connected cells; widespread service impact | Huawei RNC V600R008 |
| 13 | Cell Availability | Availability | ≥ 99.5% | >= | 1 | ≥ 99.5% | ≥ 99% | Percentage of time cell is available for service | Cell unavailability impacts coverage; users in that area lose service | Huawei RNC V600R008 |
| 14 | HSDPA Throughput per User | HSDPA Throughput per user (Local Cell)(Kbps) | ≥ 1000 Kbps | >= | 1 | ≥ 1000 Kbps | ≥ 800 Kbps | Average HSDPA throughput per user | Low throughput impacts user experience; slow data speeds | Huawei RNC V600R008 |

4. LTE (4G) Complete Threshold Table

4.1 Accessibility KPIs (🚪)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | RRC Connection Success Rate | RRC Setup Success Rate(%) | ≥ 98.7% | >= | 2 | ≥ 98.7% | ≥ 98% | Percentage of successful RRC connection establishments between UE and network | UEs cannot establish radio connection; users cannot access any LTE service | 3GPP TS 36.331 |
| 2 | E-RAB Establishment Success Rate | E-RAB Setup Success Rate | ≥ 98.7% | >= | 2 | ≥ 98.7% | ≥ 98% | Percentage of successful Evolved Radio Access Bearer setups for data transmission | UEs cannot establish radio bearer; users cannot transmit/receive data | 3GPP TS 36.413 |
| 3 | CSFB Preparation Success Rate | CSFB Preparation Success Rate | ≥ 98% | >= | 2 | ≥ 98% | ≥ 96% | Success rate of preparing Circuit Switched Fallback from LTE to 2G/3G for voice calls | Voice calls (CSFB) fail to establish; users cannot make/receive voice calls on LTE | 3GPP TS 36.413 |
| 4 | S1-Signal Connection Establishment SR | S1-Signal Connection Establishment Success Rate (%)(%) | ≥ 97% | >= | 1 | ≥ 97% | ≥ 95% | Percentage of successful S1 setup procedures between eNodeB and EPC | UEs cannot establish connection to core network; cannot access network services | 3GPP TS 36.413 |
| 5 | VoLTE Setup Success Rate | VoLTE Setup Success Rate-ZM(%) | ≥ 99% | >= | 1 | ≥ 99% | ≥ 98% | Success rate of VoLTE call establishment | VoLTE calls fail; VoLTE users cannot make high-quality voice calls | Huawei eNodeB V100R008 |

4.2 Retainability KPIs (🔗)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Call Drop Rate (All) | Service Drop Rate (All) | ≤ 1% | <= | 2 | ≤ 1% | ≤ 1.5% | Percentage of services (voice/data) dropped after successful establishment | Frequent call/data session interruptions; poor service retainability | 3GPP TS 36.331 |
| 7 | Downlink Packet Loss Rate | Downlink Packet Loss | ≤ 0.5% | <= | 1 | ≤ 0.5% | ≤ 0.8% | Percentage of data packets lost in the downlink direction | Buffering, slow browsing, poor streaming; poor downlink quality | 3GPP TS 36.323 |
| 8 | Uplink Packet Loss Rate | Uplink Packet Loss Rate | ≤ 0.5% | <= | 1 | ≤ 0.5% | ≤ 0.8% | Percentage of data packets lost in the uplink direction | Slow uploads, poor voice/video call quality; poor uplink quality | 3GPP TS 36.323 |
| 9 | VoLTE Traffic Volume | VoLTE Traffic Volume (Erl) | ≥ 0.1 Erl | >= | 1 | ≥ 0.1 Erl | ≥ 0.05 Erl | Amount of VoLTE traffic carried | Low VoLTE adoption or poor VoLTE performance; may indicate coverage issues | Huawei eNodeB V100R008 |

4.3 Mobility KPIs (🚶)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Intra-Frequency Handover Success Rate | Intra-HO execute Success Rate | ≥ 98% | >= | 2 | ≥ 98% | ≥ 96% | Success rate of handover between cells on the same frequency | Service drops during mobility within same frequency layer; poor mobility experience | 3GPP TS 36.331 |
| 11 | Inter-Frequency Handover Success Rate | Inter-Freq Handover Success Rate (%)(%) | ≥ 97% | >= | 1 | ≥ 97% | ≥ 95% | Success rate of handover between cells on different frequencies | Service disruptions when moving between frequency layers; essential for spectrum efficiency | 3GPP TS 36.331 |

4.4 Resource Utilization KPIs (📊)

| # | KPI Name | Column Name | Threshold | Operator | Weight | Whole Network Target | Cell Level Target | Definition | Impact if Violated | Huawei Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | DL PRB Utilizing Rate | DL PRB Utilizing Rate(%) | ≤ 70% | <= | 1 | ≤ 70% | ≤ 80% | Downlink Physical Resource Block utilization | Congestion; impacts user throughput and accessibility | 3GPP TS 36.211 |
| 13 | UL PRB Utilizing Rate | UL PRB Utilizing Rate(%) | ≤ 60% | <= | 1 | ≤ 60% | ≤ 70% | Uplink Physical Resource Block utilization | Congestion; impacts uplink performance and VoLTE quality | 3GPP TS 36.211 |
| 14 | User DL Average Throughput | User Downlink Average Throughput (Mbps) | ≥ 5 Mbps | >= | 1 | ≥ 5 Mbps | ≥ 3 Mbps | Average downlink throughput per user | Low throughput impacts user experience; perceived as slow data speeds | Huawei eNodeB V100R008 |
| 15 | User UL Average Throughput | User Uplink Average Throughput (Mbps) | ≥ 1 Mbps | >= | 1 | ≥ 1 Mbps | ≥ 0.8 Mbps | Average uplink throughput per user | Low uplink impacts VoLTE quality, file uploads, video calls | Huawei eNodeB V100R008 |
| 16 | Radio Network Availability Rate | Radio Network Availability Rate(%) | ≥ 99.5% | >= | 1 | ≥ 99.5% | ≥ 99% | Percentage of time radio network is available for service | Low availability impacts coverage; users lose service in affected areas | Huawei eNodeB V100R008 |
| 17 | L.UL.Interference.Avg | L.UL.Interference.Avg(dBm) | ≥ -110 dBm | >= | 1 | ≥ -110 dBm | ≥ -115 dBm | Average uplink interference level (higher = better) | High interference impacts VoLTE quality, packet loss, call drop rates | Huawei eNodeB V100R008 |
| 18 | L.Traffic.User.Avg | L.Traffic.User.Avg | ≥ 1 | >= | 1 | ≥ 1 | ≥ 0.5 | Average number of active users in the cell | Low user count may indicate coverage issues or cell is poorly located | Huawei eNodeB V100R008 |
| 19 | L.Traffic.User.Max | L.Traffic.User.Max | ≥ 5 | >= | 1 | ≥ 5 | ≥ 3 | Maximum number of active users in the cell | Indicates peak capacity demand; used for capacity planning | Huawei eNodeB V100R008 |

5. Technology Comparison Summary

5.1 KPI Count by Technology & Dimension

| Technology | Accessibility | Retainability | Mobility | Resource | Total KPIs | Weight=2 KPIs |
| --- | --- | --- | --- | --- | --- | --- |
| GSM (2G) | 6 | 2 | 3 | 8 | 19 | 2 (Call Setup SR, TCH Drop Rate) |
| UMTS (3G) | 4 | 2 | 5 | 3 | 14 | 2 (RRC Setup SR, CS Drop Rate) |
| LTE (4G) | 5 | 4 | 2 | 8 | 19 | 4 (RRC Setup SR, E-RAB Setup SR, CSFB Prep SR, Service Drop Rate) |
| Total | 15 | 8 | 10 | 19 | 52 | 8 |

5.2 Priority KPI Summary (All Technologies)

| Priority | KPI | Technology | Dimension | Weight | Threshold |
| --- | --- | --- | --- | --- | --- |
| 🔴 Critical | Call Setup Success Rate - CS | GSM | Accessibility | 2 | ≥ 98% |
| 🔴 Critical | TCH Call Drop Rate | GSM | Retainability | 2 | ≤ 1% |
| 🔴 Critical | RRC Setup Success Rate | UMTS | Accessibility | 2 | ≥ 98.7% |
| 🔴 Critical | CS Call Drop Rate | UMTS | Retainability | 2 | ≤ 1% |
| 🔴 Critical | RRC Connection Success Rate | LTE | Accessibility | 2 | ≥ 98.7% |
| 🔴 Critical | E-RAB Establishment Success Rate | LTE | Accessibility | 2 | ≥ 98.7% |
| 🔴 Critical | CSFB Preparation Success Rate | LTE | Accessibility | 2 | ≥ 98% |
| 🔴 Critical | Service Drop Rate | LTE | Retainability | 2 | ≤ 1% |

8.3 Escalation Rules

| Rule | Description | Action |
| --- | --- | --- |
| Rule 1 | Any Weight=2 KPI fails at cell level | 🔴 Critical alert |
| Rule 2 | 2G Call Setup SR < 95% at network level | 🔴 Critical alert |
| Rule 3 | 2G TCH Drop Rate > 1.5% at network level | 🔴 Critical alert |
| Rule 4 | 3G RRC Setup SR < 98% at network level | 🔴 Critical alert |
| Rule 5 | 3G CS Drop Rate > 1.5% at network level | 🔴 Critical alert |
| Rule 6 | 4G RRC Setup SR < 98% at network level | 🔴 Critical alert |
| Rule 7 | 4G Service Drop Rate > 1.5% at network level | 🔴 Critical alert |
| Rule 8 | Same cell appears in Top 10 for 3+ consecutive days | 🟡 Major alert (chronic issue) |
| Rule 9 | 5+ cells fail same Weight=1 KPI | 🟡 Major alert (systemic issue) |
| Rule 10 | Data freshness > 2 days | 🟡 Major alert (data pipeline) |

✅ Summary

This comprehensive guide provides:

| Item | Count |
| --- | --- |
| Total KPIs documented | 52 |
| GSM KPIs | 19 |
| UMTS KPIs | 14 |
| LTE KPIs | 19 |
| Weight=2 KPIs | 8 |
| Dimensions | 4 |
| Threshold tiers | Whole Network + Cell Level |
| Data validation rules | 6 types |
| Escalation levels | 4 |
| Alerting rules | 10 |